床垫申请BS 7177应选择什么危险级别测试？

BS 7177是英国针对床垫、床架等家具阻燃性能的核心标准，其核心目标是通过不同危险等级的划分，确保产品在特定使用场景下的防火安全性。根据标准要求，床垫的测试等级需根据其最终用途选择对应的危险级别。

标准将危险等级划分为低危、中危、高危和极高危四个等级：

| 危险等级 | 测试方法 | 使用场景 |
| --- | --- | --- |
| 低危等级 | BS EN 597-1、BS EN 597-2 | 家庭住宅（（包括非机动大篷车） |
| 中危等级 | BS EN 597-1、BS EN 597-2、BS 6807 | 寄宿学校、日间中心、大学和学院宿舍、度假营小屋、医院、旅馆、酒店、养老院、寄宿学校、服务食堂 |
| 高危等级 | BS EN 597-1、BS EN 597-2、BS 6807 | 某些医院病房、旅馆、酒店、海上设施、养老院 |
| 极高等级 | BS EN 597-1、BS EN 597-2、BS 6807 | 封闭式精神病房、监狱牢房 |

根据上表建议：

• 家庭用途：选择低危险级别，满足常规防火要求

• 酒店/公共场所：强制选择中危险级别，因涉及公共安全法规

• 医院/监狱：需根据具体区域选择高或极高危险级别，例如医院病房需通过高危险测试

另外需要注意的是交通工具上的软垫座椅本标准不适用。

火源类型与测试强度：

不同的危险级别，测试标准不同，点火源不同

• 低危险级别火源：阴燃香烟（BS EN 597-1）、模拟火柴火焰（BS EN 597-2）

• 中危险级别火源：低危险的所有火源、松木火种5

• 高危险级别火源：低危险的所有火源、松木火种7

• 极高危险级别火源：高危险的所有火源、附加火源

What hazard level test should be selected for mattress application to BS 7177?

BS 7177 is the UK's core standard for the flame retardancy of furniture such as mattresses and bed frames. Its core goal is to ensure the fire safety of products in specific usage scenarios through the classification of different hazard levels. According to the standard requirements, the test level of the mattress must select the corresponding hazard level according to its final use.

| 危险等级 | 测试方法 | 使用场景 |
| --- | --- | --- |
| 低危等级 | BS EN 597-1、BS EN 597-2 | 家庭住宅（（包括非机动大篷车） |
| 中危等级 | BS EN 597-1、BS EN 597-2、BS 6807 | 寄宿学校、日间中心、大学和学院宿舍、度假营小屋、医院、旅馆、酒店、养老院、寄宿学校、服务食堂 |
| 高危等级 | BS EN 597-1、BS EN 597-2、BS 6807 | 某些医院病房、旅馆、酒店、海上设施、养老院 |
| 极高等级 | BS EN 597-1、BS EN 597-2、BS 6807 | 封闭式精神病房、监狱牢房 |

The standard divides the risk level into four levels: low risk, medium risk, high risk and extremely high risk:

According to the above table, it is recommended that:

• Home use: select low risk level to meet general fire protection requirements

• Hotel/public places: mandatory selection of medium risk level, because it involves public safety regulations

• Hospital/prison: need to select high or very high risk level according to specific areas, for example, hospital wards need to pass high risk test

Also note that this standard does not apply to cushioned seats on vehicles.

Fire source type and test intensity:

Different risk levels, different test standards, different ignition sources

• Low risk level fire source: smoldering cigarette (BS EN 597-1), simulated match flame (BS EN 597-2)

• Medium risk level fire source: all low risk fire sources, pine wood fire 5

• High risk level fire source: all low risk fire sources, pine wood fire 7

• Very high risk level fire source: all high risk fire sources, additional fire sources